Landsat Toolbox for ArcGIS 10.1 User’s Manual

Tom Dilts ● 775-784-1447 ● tdilts@cabnr.unr.edu
Great Basin Landscape Ecology Lab
Department of Natural Resources and Environmental Science
University of Nevada Reno
http://www.cabnr.unr.edu/weisberg/
http://gislandscapeecology.blogspot.com/
8/14/2015

Purpose

The Landsat Toolbox for ArcGIS provides many basic preprocessing tools that can be used to help facilitate change detection and vegetation dynamics studies. This toolbox lessens the need for commercial remote sensing packages, such as ENVI or ERDAS, and brings some image processing functionality directly into ArcMap. Image pre-processing involves steps that may be under-appreciated by some GIS analysts, but are nonetheless important for ensuring reliable outcomes. This toolbox contains tools to do the following:

1) Convert raw DN values to top-of-atmosphere reflectance
2) Perform radiometric normalization using user-selected pseudo-invariant pixels
3) Perform topographic corrections using a digital elevation model
4) Mosaic adjacent scenes using linear regression to ensure a smooth edge-match

Many of the tools in this toolbox require fmask or fmask for R to perform cloud, cloud shadow, and snow masking prior to running. However, you could also do the masking manually by setting any value that you wish to remove to > 0.

https://code.google.com/p/fmask/
https://bitbucket.org/droletg/fmask

Tools

1) Convert raw DN values to top-of-atmosphere reflectance based on http://landsat.usgs.gov/Landsat8_Using_Product.php

Why? – Raw DN values are of limited value while reflectance is a much more useful and standard measurement. Top-of-atmosphere is a measurement that can be compared across sensors and dates. I currently recommend Steve Kochaver’s tool for Landsat 4 – 7- http://www.arcgis.com/home/item.html?id=3c0248c3b5d04be39f0faf334491f235.

Parameters:

1) Input Landsat workspace – Folder where the landsat files reside
2) Input metadata file – Text file with the Landsat metadata
3) Input scratch workspace – Workspace to store temporary outputs
4) Output raster – Output raster corrected to top-of-atmosphere reflectance

2) Topographic correction using linear regression on a hillshade

Why? – Steep topography can result in differential illumination on south versus north facing slopes. Slope angle is important as well since slopes perpendicular to the sun angle are going to be brighter than those at more obtuse angles. This fairly simple correction method involves using a digital elevation model (DEM) and linear regression to predict reflectance for a pixel based on the known sun angle and azimuth at the time of satellite overpass. More illuminated slopes are adjusted downward while less illuminated slopes are adjusted upward. I’ve found topographic correction to be an essential image pre-processing step in topographically heterogeneous environments. This tool uses the hillshade function in ArcGIS allowing for shadows to be cast.

Parameters:

1) Input scratch workspace – Workspace to store temporary outputs
2) Input digital elevation model – Digital elevation model of approximately the same resolution as the Landsat imagery
3) Input points – A point shapefile that could be random points or systematic points. The number of points represents a trade-off between accuracy and 
4) Input Fmask raster – Raster with clouds, cloud shadows, and snow masked out (value > 0 denote that the pixels should not be considered in topographic correction)
5) Input reflectance raster - Raster corrected to top-of-atmosphere reflectance
6) Input metadata file - Text file with the Landsat metadata
4) Output raster – Output raster topographically corrected

3) Radiometric normalization using pseudo-invariant points

Why? – Image brightness can vary by date due to non-surface effects, including atmospheric effects. Radiometric normalization attempts to adjust one or more images to a master image in order to make all images more comparable. Radiometric normalization is most valuable in environments with high atmospheric water vapor content. This tool uses user-selected pseudo-invariant targets. Selection of pseudo-invariant targets should be done with care. Pseudo-invariant targets should be flat areas, non-vegetated, subject to very little change over time, and should cover a range of brightness values. Selection of inappropriate pseudo-invariant targets can actually make the resulting normalized image worse than the original.

Parameters:

1) Input scratch workspace – Workspace to store temporary outputs
2) Input master image – Image that the second image will be matched to spectrally
3) Input second image – Image that will be adjusted to match the master image
4) Input master mask – Fmask output for the master image
5) Input second mask - Fmask output for the second image
6) Input pseudo-invariant points – Points representing locations that do not change
7) Output corrected image – Output image that has been radiometrically adjusted

4) Mosaicking with normalization using linear regression

Why?- Standard mosaicking tools in ArcGIS almost always introduce visually-apparent seamlines because of minor differences in brightness among adjacent images. This tool adjusts a second image to a master image using the overlapping area. Values from each band are extracted and linear regression is used to adjust the second image to match the first image as closely as possible. The fmask raster is needed to exclude pixels obscured by clouds, cloud shadows, and snow.

Parameters:

1) Input scratch workspace – Workspace to store temporary outputs
2) Input master mask – Fmask output for the master image
3) Input second mask - Fmask output for the second image
2) Input master image - Image that the second image will be matched to spectrally
3) Input second image - Image that will be adjusted to match the master image
5) Output mosaicked image - Output mosaicked image

Examples

Figure 1: Each band in the Landsat folder is stored as an individual TIFF file. There is an associated metadata file (_MTL.txt) that contains important parameters such as sun angle and azimuth at the time of satellite overpass.

Figure 2: Color infrared composite image with water (green), cloud shadows (brown), snow (light blue), cloud (blue), and border (pink) mask overlaid. The identify button shows that this image has been converted from raw DN to top-of-atmosphere reflectance and composited to make a six band TIFF image. The 4,3,2 false-color composite shows red channel = NIR band, green channel = red band, and blue channel = green band.

Figure 3: False color composite (NIR, R, G) before (left) and after (right) topographic correction. The post-topographic correction image appears flat.

Figure 4: Standard mosaicking tools in ArcMap can produce seamlines that are visually apparent (top). The mosaic with normalization tool adjusts a second image to the master image using linear regression. The result is a smooth continuous mosaic with no visual edge effects. Typically, the correlation coefficients between the two images is > 0.9 and the adjustment to the value of the second image is minor.