## Tutorial Overview and Getting Started:

Wild Planner contains a suite of tools designed to assist with planning for development that protects wildlife resources.  This tutorial will guide the user through each of the available tools with typical examples for their application.  This tutorial requires ArcGIS 9.x or higher with an ArcInfo and Spatial Analyst license.  It is recommended that the contents of the Wild Planner disc are copied to a local or network hard drive before staring the tutorial.  Make sure you copy the entire directory structure intact to ensure proper function of the tools and tutorial.

Starting the Tutorial:

To start the tutorial, navigate to the ‘Wild Planner Tutorial’ folder within the Wild Planner directory tree and double click on the ‘Wild Planner Tutorial.mxd’ file to open the tutorial project in ArcMap.  Alternatively, you can open ArcMap and navigate to the tutorial map (.mxd) file to open it.

Once the tutorial map file is open, you will see the base layers for the tutorial.  These include a black boundary around the Madison Valley, Montana; a shaded relief, and gray areas indicating forest service lands.  These base layers are intended only to help the user orient themselves within the tutorial area and to assist with visualization.  These layers may be turned off as desired.

How the Tutorial is Arranged:

Layers are grouped by lesson in the Table of Contents (far left panel).  Clicking the checkbox next to a lesson title turns on the layers within the lesson in the map viewer.  Clicking the ‘+’ sign next to the lesson title will expand the lesson to reveal all of the layers contained within it.  These layers may be turned on or off in the map viewer independently by checking or unchecking boxes next to each layer’s name.  Just make sure the lesson’s group layer is also checked if you want to see a layer in the map viewer.

Some layers are used in more than one lesson and are duplicated in each lesson where they are used.  This is for convenience to allow the user to easily view layers associated with each lesson.  If a layer is duplicated in more than one lesson, it doesn’t matter which copy is used for input to a tool.  For example, if you are instructed to select the ‘mad_valley_constraint_layer’ as input for a tool in Lesson 4, it doesn’t matter if you select the layer from Lesson 1 or Lesson 4.  The result will be the same.

The Wild Planner toolbox is located in the ArcToolbox window of the map project.  It will probably be located at, or near, the bottom of the toolbox list depending on what other toolboxes are loaded on you particular ArcMap installation.  Tools may be revealed or concealed but clicking on the ‘+’ or ‘-‘ symbols next to item names.  The tools are arranged in three groups: Landscape Evaluation Tools, Policy Tools, and Utilities. When a tool is opened, context sensitive help is displayed in the right-hand panel.  The complete help file can be opened by clicking the ‘Tool Help’ button at the bottom of the tool dialogue box.  For more information about how the tools work, see ‘Wild Planner Documentation.pdf’ included with this package.

List of Sample Inputs:

Item 1

## Lesson 1: Determine Appropriate Development Density

This lesson demonstrates how Wild Planner can be used to determine appropriate housing densities to maintain a desired amount of wildlife core or movement habitat.  The first part of this lesson calculates suitable densities for maintaining secure habitat cores for elk.  The second part of the lesson calculates density for maintaining general movement permeability for grizzly bears in the southern portion of the valley.  Note: The tools used in this lesson assume random placement of structures within the analysis area. This results in lower calculated densities than would be required if development were clustered or strategically placed to minimize disturbance to existing habitat.  The tools provide a simulation of regulating density without clustering or habitat setback requirements or incentives.  Future enhancement to the tools to allow clustering simulations is planned.

Lesson 1a: Find Target Density for Habitat Cores

- Check the box next to Lesson 1 to turn the layers on and click the ‘+’ symbol to expand the contents.

  - In this example, green areas represent potential elk winter range, pink represents buildable areas (private land with no conservation easements and no steep slopes), existing roads are dark gray lines.

- Navigate to the Wild Planner Policy Tools and double-click ‘Calculate Target Housing Density’.

  - Click the folder icon to the right of the ‘Output Table’ box and navigate to the ‘Wild Planner\Tutorial\TutorialOutput’ folder (or preferred folder to store tutorial outputs) and type ‘coredensity.dbf’.

  - Change ‘Number of Replications’ to 2 (note: this is only to speed up processing for the demonstration.  Increasing the number of replication increases the accuracy of results.  The default value of 10 replications is typically suitable for actual application).

  - Click the drop-down arrow next to ‘Analysis Extent’ and select ‘elk_analysis_area’ (This is the area containing habitat that you want to conserve).

  - For ‘Constraint Layer’, select ‘Mad_Valley_constraint_layer’ (This is the area that can actually be built upon).

  - For ‘Existing Roads’, select ‘mv_roads_2000’.

  - For ‘Elevation Raster’, select ‘mad_dem’.

  - Enter 250 for ‘Minimum Habitat Patch’ and select ‘Acres’ for units.

  - Enter 0.5 for ‘Structure Influence Distance’ and select ‘Miles’ for units.

  - Enter 100 for ‘Road influence Distance’ and select ‘Yards’ for units.

  - Change ‘Percent Area to Conserve’ to 50.  This instructs the tool to find the density of randomly placed houses that will maintain 45-55% of the existing habitat as secure (core) habitat.  We have defined secure habitat (from our tool inputs) as habitat patches that are at least ½ mile from a house, 100 yards from a road, and at least 250 contiguous acres in size.

  - For ‘Output Units’, select acres.

  - After reviewing to make sure all of the inputs are correct, click ‘OK’ to run the tool.

  - When the tool finds a density that satisfies the conservation target, a window appears display the final calculated density.  Two density values are giving: the density of houses within the buildable area, and the density of houses within the analysis area.  After reviewing the information, click OK to continue.

  - When the tool completes, output from the final replication that met the target criteria are displayed in the map viewer.  The results of each replication are stored in the output table (‘coredensity.dbf’) and is added to the table of contents.

Lesson 1b: Find Density for Movement Habitat

- In the Lesson 1 table of contents, right-click on ‘southern_corridor’ and selct ‘Zoom to Layer’.  Make sure the corridor layer is turned on.

- Navigate to the Wild Planner Policy Tools and double-click ‘Calculate Corridor Housing Density’.

  - Click the folder icon to the right of the ‘Output Table’ box and navigate to the ‘Wild Planner\Tutorial\TutorialOutput’ folder (or preferred folder to store tutorial outputs) and type ‘corridordensity.dbf’.

  - Change ‘Number of Replications’ to 2 (note: this is only to speed up processing for the demonstration.  Increasing the number of replication increases the accuracy of results.  The default value of 10 replications is typically suitable for actual application).

  - Click the drop-down arrow next to ‘Analysis Extent’ and select ‘southern_corridor’ (This is the area containing habitat that you want to conserve).

  - For ‘Constraint Layer’, select ‘Mad_Valley_constraint_layer’ (This is the area that can actually be built upon).

  - Enter 200 for ‘Minimum Corridor Width’ and select ‘Yards’ for units.

  - Enter 500 for ‘Disturbance Distance’ and select ‘Yards’ for units.

  - For ‘Output Units’, select acres.

  - After reviewing to make sure all of the inputs are correct, click ‘OK’ to run the tool.

  - When the tool finds a density that satisfies the conservation target, a window appears display the final calculated density.  Two density values are giving: the density of houses within the buildable area, and the density of houses within the analysis area.  After reviewing the information, click OK to continue.

  - When the tool completes, output from the final replication that met the target criteria are displayed in the map viewer.  The results of each replication are stored in the output table (‘corridordensity.dbf’) and is added to the table of contents.

- When you are finished exploring the outputs, uncheck the box next to Lesson 1 in the table of contents to turn the layers off and turn off the 3 output layers.

## Lesson 2: Evaluate Habitat Core

This lesson demonstrates how Wild Planner can be used to determine the pattern of habitat cores on the landscape.  This lesson evaluates the current pattern of core habitat for elk based on existing roads and structures.  Lesson 5 demonstrates how these tools can be used to evaluate the impact of hypothetical development scenarios on wildlife habitat.

- Check the box next to Lesson 2 to turn the layers on and click the ‘+’ symbol to expand the contents.

  - In this example, existing structures are represented as black house-shaped points and existing roads are dark gray lines.

- Right-click on ‘elk_analysis area’ and select ‘Zoom to Layer’.

- Navigate to the Wild Planner Landscape Evaluation Tools and double-click ‘Evaluate Habitat Patches’.

  - For ‘Analysis Extent’, select ‘elk_analysis_area’.

  - For ‘Structures Layer’, select ‘Structures_090208’.

  - For ‘Roads Layer’, select ‘mv_roads_2000’.

  - Enter the same parameters used in Lesson 1, step 2 for patch size, influence distances, and output units.

  - Click on the navigation folder next to ‘Output Shapefile’ and navigate to the Tutorial Output folder.  Type ‘existing cores’ in the ‘Name’ box.

  - Click ‘OK’ to run the tool.

  - A window will be displayed indicating the percentage of available habitat that satisfies the criteria for “core” habitat (areas at least 100 yards from a road, ½ mile from a house, and at least 250 contiguous acres in size).  Click ‘OK’ to continue.

- When the tool completes, a shapefile containing the results will be displayed in the map viewer.  Beige areas indicate undisturbed habitat that is too small to be considered a core.  Green areas represent core habitat.

- When you are finished exploring the outputs, uncheck the box next to Lesson 2 in the table of contents to turn the layers off and turn off the output layer.

## Lesson3: Evaluate Movement Habitat

This lesson demonstrates how Wild Planner can be used to identify habitat linkages or “corridors” in the landscape.  This lesson evaluates the effect of the current pattern of development on potential grizzly bear movement in the southern part of the valley.  Lesson 5 demonstrates how these tools can be used to evaluate the impact of hypothetical development scenarios on wildlife habitat movement.  The first part of the lesson identifies potential linkage areas.  The second part ranks linkage areas by the relative level of security they are predicted to provide.  Lesson 5 demonstrates how these tools can be used to evaluate the impact of hypothetical development scenarios on wildlife habitat movement.

Lesson 3a: Find Potential Linkage Areas

- Check the box next to Lesson 3 to turn the layers on and click the ‘+’ symbol to expand the contents.

  - In this example, existing structures are represented as black house-shaped points and the yellow area is a potential connectivity zone for grizzly bears moving between the Madison and Gravelly Ranges.

- Right-click on ‘southern_corridor’ and select ‘Zoom to Layer’.

- Navigate to the Wild Planner Landscape Evaluation Tools and double-click ‘Evaluate Movement Landscape’.

  - For ‘Structures Layer’, select ‘Structures_090208’.

  - Enter 200 for ‘Minimum Corridor Width’ and select ‘Yards’ for units.

  - Enter 500 for ‘Disturbance Distance’ and select ‘Yards’ for units.

  - Click on the navigation folder next to ‘Output Raster’ and navigate to the Tutorial Output folder.  Type ‘existlink’ in the ‘Name’ box.

  - Leave ‘Analysis Cell Size’ at the default value.

  - Set ‘Analysis Extent to ‘Same As southern_corridor’.

  - Check the ‘Consider Viewshed’ box to consider areas hidden by topography from structures as potential linkage regardless of whether it falls within the disturbance zone.

  - For ‘Elevation Raster’, select ‘mad_dem’.

  - Click ‘OK’ to run the tool.

- Drag the ‘existlink’ layer in the table of contents below the structures layer in the ‘Lesson 3’ group and examine the output.  Blue areas represent areas of potential movement habitat (areas at least 200 yards wide that are at least 500 yards from a structure).  Orange areas represent areas likely to be avoided (areas within 500 yards of a structure or where the undisturbed area is less than 200 yards wide).

Lesson 3b: Rank Potential Linkage Zone Quality

- To evaluate the quality of linkage areas created in the previous step, navigate to the Wild Planner Landscape Evaluation Tools and double-click ‘Rank Linkage Zones’.

  - For ‘Structures Layer’, select ‘Structures_090208’.

  - For ‘Land Cover Layer’, select ‘Montana Landcover 2010’.

  - Leave ‘Species Habitat Preference’ at the default value (FOREST-SHRUB).

  - Click on the navigation folder next to ‘Output Raster’ and navigate to the Tutorial Output folder.  Type ‘existlink’ in the ‘Name’ box.

  - For ‘Linkage Layer’, select the linkage raster you just created (‘existlink’).

  - Under ‘Movement Direction to Analyze’, check the box next to ‘East-West’.

  - Click on the navigation folder next to ‘Output Raster’ and navigate to the Tutorial Output folder.  Type ‘ranklink’ in the ‘Name’ box.

  - Click ‘OK’ to run the tool.

- Drag the ‘ranklink’ layer in the table of contents below the structures layer and above the ‘existlink’ layer in the ‘Lesson 3’ group and examine the output. Dark blue areas represent the highest quality linkage areas where structure density is low and/or the preferred habitat (forest and shrub) is abundant.  Dark brown areas are the worst linkage quality where structure density is high and preferred habitat is rare.

- When you are finished exploring the outputs, uncheck the box next to Lesson 3 in the table of contents to turn the layers off and turn off the output layer.

## Lesson 4: Construct Simulations

In this lesson you will use the utility tools to construct build-out scenarios for evaluation.  Lesson 4a provides a simple example of how to generate a simulated road network that connects hypothetical or planned structures to existing roads.  Lesson 4b demonstrates how to generate a full build-out scenario based on the existing parcels.

Lesson 4a: Generate Roads to Hypothetical Structures

- Check the box next to Lesson 4 to turn the layers on and click the ‘+’ symbol to expand the contents.

  - In this example, existing structures are represented as black house-shaped points, existing roads are dark gray lines, and pink represents buildable areas.  Existing parcel boundaries are represented as thin black lines.

- Right-click on ‘Simulated Development’ and select ‘Zoom to Layer’.  Zoom out from this point (using the ‘Fixed Zoom Out’ tool bar button) until you can see parcel boundary lines.  You will see a representation of seven proposed structures in the northwest corner of a parcel with a single existing road entering the parcel.

- Navigate to the Wild Planner Utilities and double-click ‘Generate Roads’.

  - For ‘Elevation Raster’, select ‘mad_dem’.

  - For ‘Slope Raster’, select ‘mad_slope (note that if a slope raster is not selected, it will be generated for you).

  - For ‘Existing Roads’, select ‘mv_roads_2000’.

  - For ‘New Structure Layer’, select ‘SimulatedStructures’.

  - Click on the navigation folder next to ‘New Roads Output Layer’ and navigate to the Tutorial Output folder.  Type ‘Simulated Roads’ in the ‘Name’ box.

  - Leave ‘Append to Existing Roads’ checked.

  - Set ‘Analysis Extent to ‘Same as Display’.

  - Click ‘OK’ to run the tool.

- The tool generates roads that connect each structure to an existing road by balance distance and gradient (roads are the shortest distance along the lowest gradient possible).  Red roads represent the new simulated roads and black roads are existing roads.

- When you are finished examining the output, turn off the Simulated Roads layer.

Lesson 4b: Simulating Full Build-out

- Right-click on ‘mad_bndry’ near the top of the table of contents and select ‘Zoom to Layer’.

- Navigate to the Wild Planner Utilities and double-click ‘Simulate Buildout’.

  - For ‘Parcel Layer’, select ‘mv_parcels’.

  - For ‘Buildable Areas’, select ‘Mad_valley_constraint_layer’.

  - For ‘Existing Structures’, select ‘Structures_090208’.

  - For ‘Existing Roads’, select ‘mv_roads_2000’.

  - For ‘Elevation Raster’, select ‘mad_dem’.

  - Set ‘Analysis Extent to ‘Same as layer ‘mad_bndry’’.

  - Click on the navigation folder next to ‘Output Structures’ and navigate to the Tutorial Output folder.  Type ‘Full Buildout Structures; in the ‘Name’ box.

  - Click on the navigation folder next to ‘Output Roads’ and navigate to the Tutorial Output folder.  Type ‘Full Buildout Roads; in the ‘Name’ box.

  - Click ‘OK’ to run the tool.

- Drag the ‘Full Buildout Structures’ and ‘Full Buildout Roads’ layers to the top of the Lesson 4 table of contents.

- Zoom in to examine the output.  Black structures are existing and gray structures are new structures that could potentially be built.  Existing roads are represented as black lines and simulated roads that connect potential new structures to existing roads are represented as red lines.

## Lesson 5: Evaluate Simulated Development

This lesson demonstrates how Wild Planner can be used to evaluate the potential impact of simulated development and compare the results with current conditions.

- Repeat Lessons 2-3 except whenever you are asked for a structures or roads input, use the full build-out outputs generated in Lesson 4.

- As each tool is executed, drag the output into the Lesson 4 table of contents just below the simulated build-out outputs generated in Lesson 4.